# TARURA / SAKIRP Checklist bridges

Supervisor:

Bridge:

Date:

| Issue | Observation on site | yes | no |
| --- | --- | --- | --- |
| Respect dimensions | Are dimensions abutments correct |  |  |
|  | Are abutments aligned and square |  |  |
| Quality sand delivered | Low silt / clay content |  |  |
|  | Is there sufficient sand on site |  |  |
|  | Are drainage channels dug to avoid erosion of sand heaps |  |  |
| Quality stones | Are stones larger and strong |  |  |
|  | Are smooth stones roughed up with grinder |  |  |
|  | Are stones washed away from actual construction and with clean water |  |  |
|  | Do washed stones remain clean – i.e. they are not put again on muddy surface |  |  |
|  | Are the best shaped stone put aside for the closure of the arch |  |  |
|  | Is village up to date with stone delivery? |  |  |
| Mortar mixed | Sufficient mixing – 4 times |  |  |
|  | Correct ratio 1:3 or 1:4 |  |  |
|  | Is debris/ rubbish removed from sand |  |  |
|  | Is the water ratio of the mix correct? |  |  |
|  | No mortar is wasted during transport in karai |  |  |
| Stone masonry | Are large stones used for construction? |  |  |
|  | Are smooth stones being roughed up with a grinder? |  |  |
|  | Are stones washed with a brush and water |  |  |
|  | Are corner stones of right shape and large |  |  |
|  | Are stones not wobbly i.e. they do not move or are fully embedded with mortar |  |  |
|  | Are stones laid flat to ensure strong abutment |  |  |
|  | Are stones of the right size and shape selected or is any stone just put and the void filled with mortar |  |  |
|  | Are all gaps between stones filled with mortar |  |  |
|  | Are stone crushed and used for filling gaps |  |  |
|  | Is there a good bond between the stones |  |  |
|  | Was scaffold made to avoid that craftsmen walk on fresh stone masonry? |  |  |
|  | Is the stone masonry kept damp to allow for curing |  |  |
|  | Is spilled mortar being recycled through bags on the ground |  |  |
|  | Is fresh masonry protected against the rain with tarpaulin |  |  |
| Scaffolding | Check soundness formwork |  |  |
|  | Risk collapse scaffold because floods |  |  |
|  | Are tree poles well secured |  |  |
|  | Are the trusses on wedges for easy removal form work |  |  |
|  | Are holes pre-drilled if timber is too hard |  |  |

| Issue | Observation on site | yes | no |
| --- | --- | --- | --- |
| Stone arch | Are stones put according to the radius i.e. square on circle |  |  |
|  | Are the external stones well selected i.e. do they have a nice appearance for the arch? |  |  |
|  | Are long shaped stones used for the arch |  |  |
|  | Was scaffold made to avoid that craftsmen walk on fresh stone masonry? |  |  |
|  | Is the arch closed while good shaped stones are wedged in |  |  |
|  | Is spilled mortar being recycled through bags on the form work edges |  |  |
| Use of equipment | Condition water pump |  |  |
|  | Last time that oil generator was replaced |  |  |
|  | Condition generator |  |  |
|  | Condition drill |  |  |
|  | Are the axles of wheel barrows greased |  |  |
| Store | Is cement stored on pallets |  |  |
|  | Are tools well stored |  |  |
|  | Is the ledger for cement up to date & signed |  |  |
|  | Is the ledger for stones up to date & signed |  |  |
|  | Is the ledger for sand up to date & signed |  |  |

## Tasks for supervisor at end visit

- Fill out site ledger with instruction masons

- Send picture sand / cement / stone ledger

- Send pictures to WhatsApp group

- 2 copies: TARURA and forward one to SAKIRP